Hi-Fi Speaker Wire

Product Description :

Construction :

Conductor

PVC Insulation

- OFC tinned or OFC bare , stranded copper .conductor , 26 - 22 AWG.

- Transparent or color PVC insulation.

- Braidless parallel cords.

Applications :

Uses for car speakers , general audio cable and high definition low loss speaker.

| Conductor |  |  | Jacket Thickness | Nominal O. D. |
| --- | --- | --- | --- | --- |
| AWG | No. / mm | MATRL | mm | mm |
| 26 | 7 / 0.16 | TC / BC | 0.45 | 1.4 × 2.8 |
| 24 | 11 / 0.16 | TC / BC | 0.60 | 1.8 × 3.6 |
| 22 | 17 / 0.16 | TC / BC | 0.50 | 1.8 × 3.6 |
| 20 | 26 / 0.16 | TC / BC | 0.50 | 1.9 × 3.8 |
| 18 | 41 / 0.16 | TC / BC | 0.50 | 2.2 × 4.4 |
| 16 | 26 / 0.254 | TC / BC | 0.60 | 2.5 × 5.0 |
| 14 | 41 / 0.254 | TC / BC | 0.90 | 4.1 × 8.2 |
| 12 | 65 / 0.254 | TC / BC | 1.60 | 5.8 × 11.6 |
| 14 | 15 × 7 / 0.16 | OFC | 0.90 | 4.1 × 8.2 |
| 12 | 45 × 7 / 0.12 | OFC | 1.60 | 5.8 × 11.6 |