Fluorine Resin High Temperature Cable

Construction :

Conductor

Insulation

Braid Shield

Jacket

Product Description :

  Conductor : silver coated copper clad steel

  Insulation : Fluorine resin high temperature

  Braid : silver coated copper

  Jacket : Fluorine resin high temperature

  Jacket Thickness : 0.25 ~ 0.62 mm

  Temperatre Range : -70 ºC~ 200 ºC

  Voltage Rating : 30V

  Continuous working temperature range : -70 ºC ~ 200 ºC

  Resistant to solvents, chemicals, gasoline and radiation

  High electric strength carries extremely fast transmission

  Minimal single distortion

  Tough flexible resistant to crush abrasion

Applications :

  mobile phone							  video / audio equipment					  broadcast equipment

  PC/peripherals 							  medical equipment						  identification card

  home appliances							  lighting equipment							  shopping mall information

  PDA									  remote control

  digital still camera							  wrist watch

  GPS									  car-phone

  network access device						  hands-free voice controlled device

| Style | Conductor |  | Insulation Diameter | Braid Coverage | Jacket Diameter | Capacitance | Impedance | max. VSWR Frequency/Hz |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | AWG | No. strands | (mm) | (%) | (mm) | (pF/ft) | (ohms) | 3G | 6G |
| RG 178B/U | 30 | 7 x 38 | 0.86 | 95 | 1.80 | 29.0 | 50 | 1.2 | 1.3 |
| RG 179/U | 30 | 7 x 38 | 1.58 | 95 | 2.54 | 19.5 | 75 | 1.2 | 1.3 |
| RG 316/U | 26 | 7 x 34 | 1.53 | 95 | 2.52 | 29.0 | 50 | 1.2 | 1.3 |