Category 5e 100MHz FTP Cable (Stranded)

Product Description :

Construction :

- Conductor : 24/26AWG Stranded Bare Copper

- Insulation : HDPE, FRPE

- Shield : Mylar-Aluminum/Mylar tape/tinned stranded copper braid (S-FTP)

- Drain Wire : 24AWG stranded copper

- Ripcord : Under jacket

- Jacket : FRPVC

- UL/CSA Listed CM, CMR

- UL, ETL and 3P Verified to ANSI/TIA/EIA 568-B.2 Category 5 Enhanced

- ISO/IEC-11801

- NEMA WC 63.1

- prEN 50288-2-2

- Note : Non-Plenum are CM or CMR Flame rated.

Applications :

- 10 BaseT (IEEE 802.3)

- 100 BaseT (IEEE 802.3u)

- 100 Vg-AnyLAN (IEEE 802.12)

- Token Ring (IEEE 805.5)

- TP-PMD (ANSI X3T9.5)

- 100 Mbps CDDI

- ATM 155

Cable Data

| Pairs | Jacket Color | Diameter |  | Weight per kft |  | ##### Flame Rating | Packing Put-up |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | inch | mm | lb | kg |  |  |
| 24AWG X 4P | Gray | 0.22 | 5.7 | 26.0 | 11.8 | CM | 1000' Reel-in-a-box / Reel |
| 26AWG X 4P | Gray | 0.21 | 5.4 | 21.1 | 9.6 | CM |  |

Product Electrical Characteristics

Impedance: 100 +/- 15 ohms
Mutual Capacitance (max. nf/1000ft): 17.1 
DC Resistance (max. Ohms/1000ft): 28.6 
Propagation Delay Skew: 45 nS/100M

| Frequency MHz | ATT max. | NEXT min. | ACR min. | PS.NEXT min. | PS.ACR min. | ELFEXT min. | PS.ELFEXT min. | R.L. min. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 3.2 | 65.3 | 62.1 | 62.3 | 59.1 | 63.8 | 60.8 | 20.0 |
| 4 | 6.0 | 56.3 | 50.3 | 53.3 | 47.3 | 51.7 | 48.7 | 23.0 |
| 8 | 8.5 | 51.8 | 43.3 | 48.8 | 40.3 | 45.7 | 42.7 | 24.5 |
| 10 | 9.5 | 50.3 | 40.8 | 47.3 | 37.8 | 43.8 | 40.8 | 25.0 |
| 16 | 12.1 | 47.3 | 35.2 | 44.3 | 32.2 | 39.7 | 36.7 | 25.0 |
| 20 | 13.6 | 45.8 | 32.2 | 42.8 | 29.2 | 37.8 | 34.8 | 25.0 |
| 25 | 15.2 | 44.3 | 29.1 | 41.3 | 26.1 | 35.8 | 32.8 | 24.2 |
| 31.25 | 17.1 | 42.9 | 25.8 | 39.9 | 22.8 | 33.9 | 30.9 | 23.3 |
| 62.5 | 24.8 | 38.4 | 13.6 | 35.4 | 10.6 | 27.8 | 24.8 | 20.7 |
| 100 | 32.0 | 35.3 | 3.3 | 32.3 | 0.3 | 23.8 | 20.8 | 19.0 |