Category 5e 100MHz UTP Cable (Solid)

Product Description :

Construction :

- Conductor : 24AWG Bare Solid Copper

- Insulation : HDPE, FRPE, 100% FEP

- Ripcord : Under jacket

- Jacket : FRPVC

- UL/CSA Listed CM, CMR and CMP

- UL, ETL and 3P Verified to ANSI/TIA/EIA 568-B.2 Category 5 Enhanced

- ISO/IEC-11801

- prEN 50288-3-1

- Note : Plenum are rated CMP Flame.

- Non-Plenum are CM or CMR Flame rated.

Applications :

- 10 BaseT (IEEE 802.3)

- 100 BaseT (IEEE 802.3u)

- 100 Vg-AnyLAN (IEEE 802.12)

- Token Ring (IEEE 805.5)

- TP-PMD (ANSI X3T9.5)

- 100 Mbps CDDI

- ATM 155

Cable Data

| Pairs | Jacket Color | Diameter |  | Weight per kft |  | Flame Rating | Packing Put-up (box) |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | mm | inch | lb | kg |  |  |
| 2 | Gray | 4.3 | 0.17 | 13.4 | 6.1 | CM | 1000 ft. |
| 2 | Blue | 4.3 | 0.17 | 13.4 | 6.1 | CM |  |
| 2 | Gray | 4.3 | 0.17 | 13.4 | 6.1 | CMR |  |
| 2 | Blue | 4.3 | 0.17 | 13.4 | 6.1 | CMR |  |
| 2 | Gray | 4.1 | 0.16 | 14.7 | 6.7 | CMP |  |
| 2 | Blue | 4.1 | 0.16 | 14.7 | 6.1 | CMP |  |
| 4 | Gray | 5.3 | 0.21 | 23.1 | 10.5 | CM |  |
| 4 | Blue | 5.3 | 0.21 | 23.1 | 10.5 | CM |  |
| 4 | Gray | 5.3 | 0.21 | 23.5 | 10.7 | CMR |  |
| 4 | Blue | 5.3 | 0.21 | 23.5 | 10.7 | CMR |  |
| 4 | Gray | 4.8 | 0.19 | 23.5 | 10.7 | CMP |  |
| 4 | Blue | 4.8 | 0.19 | 23.5 | 10.7 | CMP |  |

Product Electrical Characteristics

Impedance: 100 +/- 15 ohms
Mutual Capacitance (max. nf/1000ft): 17.1 
DC Resistance (max. Ohms/1000ft): 28.6 
DC Resistance Unbalance of a pair: 5% max 
Capacitance Unbalance (Pair to Ground): 330pf/100m max 
Propagation Delay Skew: 45 nS/100M

| Frequency MHz | ATT max. | NEXT min. | ACR min. | PS.NEXT min. | PS.ACR min. | ELFEXT min. | PS.ELFEXT min. | R.L. min. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2.0 | 65.3 | 63.3 | 62.3 | 60.3 | 63.8 | 60.8 | 20.0 |
| 4 | 4.1 | 56.3 | 52.2 | 53.3 | 49.2 | 51.7 | 48.7 | 23.0 |
| 8 | 5.8 | 51.8 | 46.0 | 48.8 | 43.0 | 45.7 | 42.7 | 24.5 |
| 10 | 6.5 | 50.3 | 43.8 | 47.3 | 40.8 | 43.8 | 40.8 | 25.0 |
| 16 | 8.2 | 47.3 | 39.1 | 44.3 | 36.1 | 39.7 | 36.7 | 25.0 |
| 20 | 9.3 | 45.8 | 36.5 | 42.8 | 33.5 | 37.8 | 34.8 | 25.0 |
| 25 | 10.4 | 44.3 | 33.9 | 41.3 | 30.9 | 35.8 | 32.8 | 24.3 |
| 31.25 | 11.7 | 42.9 | 31.2 | 39.9 | 28.2 | 33.9 | 30.9 | 23.6 |
| 62.5 | 17.0 | 38.4 | 21.4 | 35.4 | 18.4 | 27.8 | 24.8 | 21.5 |
| 100 | 22.0 | 35.3 | 13.3 | 32.3 | 10.3 | 23.8 | 20.8 | 20.1 |