Category 6 250MHz UTP Cable (Stranded)

Product Description :

Construction :

- Conductor : 24AWG Bare Stranded Copper

- Insulation : HDPE, FRPE

- Jacket : FRPVC

- UL/CSA Listed CM, CMR

- UL and ETL Verified to ANSI/TIA/EIA 568-B.2-1 Category 6

- ISO/IEC-11801

- NEMA WC 66

- prEN50288-6-2

- Note : Non-Plenum are CM or CMR Flame rated.

Applications :

- 1000Base T (IEEE 802.3ab)

- ATM 622 or higher

- Broadband video (77 channels, 550MHz)

Cable Data

| Pairs | Jacket Color | Diameter |  | Weight per kft |  | ##### Flame Rating | Packing Put-up |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | mm | inch | lb | kg |  |  |
| 4 | Gray | 6.0 | 0.24 | 23.5 | 10.7 | CM | 1000' Reel |
| 4 | Gray | 6.0 | 0.24 | 23.5 | 10.7 | CMR | 1000' Reel |

Product Electrical Characteristics

Impedance: 100 +/- 15 ohms
Mutual Capacitance (max. nf/1000ft): 17.1 
DC Resistance (max. Ohms/1000ft): 28.6 
DC Resistance Unbalance of a pair: 5% max 
Capacitance Unbalance (Pair to Ground): 330pf/100m max 
Propagation Delay Skew: 25 nS/100M 
Normal Velocity of Propagation: 62% 
Attenuation-to-Crosstalk Ratio: 
At 100MHz: 22.5dB/min.
At 155MHz: 13.4dB/min.
At 250MHz: 7.0dB/min.

| Frequency MHz | ATT max. | NEXT min. | ACR min. | PS.NEXT min. | PS.ACR min. | ELFEXT min. | PS.ELFEXT min. | R.L. min. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2.4 | 74.3 | 71.9 | 72.3 | 69.9 | 67.8 | 64.8 | 20.0 |
| 4 | 4.6 | 65.3 | 60.7 | 63.3 | 58.7 | 55.7 | 52.7 | 23.0 |
| 8 | 6.5 | 60.8 | 54.3 | 58.8 | 52.3 | 49.7 | 46.7 | 24.5 |
| 10 | 7.2 | 59.3 | 52.1 | 57.3 | 50.1 | 47.8 | 44.8 | 25.0 |
| 16 | 9.1 | 56.3 | 47.2 | 54.3 | 45.2 | 43.7 | 40.7 | 25.0 |
| 20 | 10.2 | 54.8 | 44.6 | 52.8 | 42.6 | 41.7 | 38.7 | 25.0 |
| 25 | 11.5 | 53.3 | 41.8 | 51.3 | 39.8 | 39.8 | 36.8 | 24.2 |
| 31.25 | 12.8 | 51.9 | 39.1 | 49.9 | 37.1 | 37.9 | 34.9 | 23.3 |
| 62.5 | 18.6 | 47.4 | 28.8 | 45.1 | 26.8 | 31.8 | 28.8 | 20.7 |
| 100 | 23.9 | 44.3 | 20.4 | 42.3 | 18.4 | 27.8 | 24.8 | 19.0 |
| 155 | 30.4 | 41.5 | 11.1 | 39.5 | 9.1 | 23.9 | 20.9 | 17.4 |
| 200 | 35.0 | 39.8 | 4.8 | 37.8 | 2.8 | 21.7 | 18.7 | 16.4 |
| 250 | 39.6 | 38.3 | 0.0 | 36.3 | NA | 19.8 | 16.8 | 15.6 |