Computer Cable (Pair Type)

UL 2464

Product Description :

Construction :

Foil Cover Type :

Braid Cover Type :

Foil Cover Type :

Conductor

SR-PVC Insulation

Jacket

- Tinned stranded , copper conductor.

- Color-coded semi-rigid PVC insulation or PVC insulation.

- Paired cores cabled under aluminum mylar shield.

- Tinned stranded copper drain wire.

PVC jacketed unpaired computer and data transmission
cable.

AL-mylar foil

Drain Wire

- Rated temperature : 80°C.
Rated voltage : 300 volts.

- Passes UL VW-1SC Vertical flame test.

Braid Cover Type :

Conductor

SR-PVC Insulation

AL-mylar foil

Jacket

Drain Wire

Braid Shielded

- Tinned stranded , copper conductor.

- Color-coded semi-rigid PVC insulation or PVC insulation.

- Paired cores cabled under aluminum mylar shield.

- Tinned stranded copper drain wire.

- Tinned copper braid shield , 85% coverage.

- PVC jacketed computer and data transmission cable.

- Rated temperature : 80°C.
Rated voltage : 300 volts.

- Passes UL VW-1SC Vertical flame test.

Applications : 

Foil Cover Type : 
Sound broadcast , audio instrumentation and computer cables for EIA RS-232 applications.

Braid Cover Type :
Computer cables for EIA RS-232 and CAD / CAM applications.

| UL Style & CUL Type | Conductor |  | No. of Pair | Insulation Thickness | Braid Shield | Jacket Thickness | Overall Diameter | Nominal Capa |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | AWG | No./mm |  | mm | No./mm | mm | mm | A | B |
| UL 2464 CUL AWM (Foil Shield) | 24 | 7 / 0.203 | 4 | 0.25 | ─ | 0.85 | 6.6 ± 0.15 | 30 | 50 |
|  |  |  | 6 |  |  | 0.85 | 7.4 ± 0.20 |  |  |
|  |  |  | 7 |  |  | 0.85 | 7.6 ± 0.20 |  |  |
|  |  |  | 8 |  |  | 0.85 | 8.3 ± 0.20 |  |  |
|  |  |  | 9 |  |  | 0.85 | 8.6 ± 0.20 |  |  |
|  |  |  | 10 |  |  | 1.02 | 9.6 ± 0.25 |  |  |
|  |  |  | 15 |  |  | 1.02 | 10.6 ± 0.30 |  |  |
|  |  |  | 19 |  |  | 1.02 | 11.6 ± 0.30 |  |  |
|  |  |  | 25 |  |  | 1.02 | 13.0 ± 0.30 |  |  |
|  |  |  | 50 |  |  | 1.02 | 17.7 ± 0.40 |  |  |
| UL 2464 CUL AWM ( Braid Shield ) | 24 | 7 / 0.203 | 2 | 0.25 | 16/8/0.127 | 0.80 | 6.3 ± 0.15 | 30 | 50 |
|  |  |  | 3 |  | 16/8/0.127 | 0.85 | 6.9 ± 0.20 |  |  |
|  |  |  | 4 |  | 24/8/0.127 | 0.85 | 7.2 ± 0.20 |  |  |
|  |  |  | 5 |  | 24/8/0.127 | 0.85 | 7.4 ± 0.20 |  |  |
|  |  |  | 6 |  | 24/8/0.127 | 0.85 | 7.7 ± 0.20 |  |  |
|  |  |  | 7 |  | 24/8/0.127 | 1.02 | 8.4 ± 0.20 |  |  |
|  |  |  | 8 |  | 24/8/0.127 | 1.02 | 8.5 ± 0.20 |  |  |
|  |  |  | 10 |  | 24/8/0.127 | 1.02 | 9.7 ± 0.25 |  |  |
|  |  |  | 12.5 |  | 24/10/0.127 | 1.02 | 10.1 ± 0.30 |  |  |
|  |  |  | 15 |  | 24/10/0.127 | 1.02 | 11.2 ± 0.30 |  |  |
|  |  |  | 18 |  | 24/10/0.127 | 1.02 | 12.1 ± 0.35 |  |  |
|  |  |  | 25 |  | 24/10/0.127 | 1.02 | 13.0 ± 0.35 |  |  |

A：Capacitance between conductors

B：Capacitance between 1 conductor and other conductors connected to shield.

Note：Capacitance is measured at 1KHz