Computer Cable

UL 2464

Product Description :

Construction :

Foil Cover Type :

Braid Cover Type :

Foil Cover Type :

Conductor

SR-PVC Insulation

AL-mylar foil

Jacket

Drain Wire

- Tinned stranded , copper conductor.

- Color-coded semi-rigid PVC insulation or PVC insulation.

- Cores cabled under aluminum mylar shield.

- Tinned stranded copper drain wire.

- PVC jacketed unpaired computer and data transmission
cable.

- Rated temperature : 80°C.

- Rated voltage : 300 volts.

- Passes UL VW-1SC Vertical flame test.

Braid Cover Type :

Conductor

SR-PVC Insulation

AL-mylar foil

Jacket

Drain Wire

Braid Shielded

- Tinned stranded , copper conductor.

- Color-coded semi-rigid PVC insulation or PVC insulation.

- Cores cabled under aluminum mylar shield.

- Tinned stranded copper drain wire.

- Tinned copper braid shield , 85% coverage.

- PVC jacketed computer and data transmission cable.

- Rated temperature : 80°C.

- Rated voltage : 300 volts.

- Passes UL VW-1SC Vertical flame test.

Applications : 

Foil Cover Type : 
Sound broadcast , audio instrumentation and computer cables for EIA RS-232 applications.

Braid Cover Type :
Computer cables for EIA RS-232 and CAD / CAM applications.

Braid Cover Type :

| UL Style & CUL Type | Conductor |  | No. of Core | Insulation Thickness | Braid Shield | Jacket Thickness | Overall Diameter | Nominal Capa |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | AWG | No./mm |  | mm | No./mm | mm | mm | A | B |
| UL 2464 CUL AWM | 28 | 7 / 0.127 | 3 | 0.25 | 16/5/0.127 | 0.80 | 4.3 ± 0.15 | 28 | 51 |
|  |  |  | 4 | 0.25 | 16/5/0.127 | 0.80 | 4.6 ± 0.15 | 28 | 51 |
|  |  |  | 5 | 0.25 | 16/6/0.127 | 0.80 | 4.8 ± 0.15 | 28 | 51 |
|  |  |  | 6 | 0.25 | 16/6/0.127 | 0.80 | 5.2 ± 0.15 | 26 | 47 |
|  |  |  | 7 | 0.25 | 16/6/0.127 | 0.80 | 5.2 ± 0.15 | 26 | 47 |
|  |  |  | 8 | 0.25 | 16/6/0.127 | 0.80 | 5.4 ± 0.15 | 26 | 47 |
|  |  |  | 9 | 0.25 | 16/6/0.127 | 0.80 | 5.7 ± 0.15 | 26 | 47 |
|  |  |  | 10 | 0.25 | 16/8/0.127 | 0.80 | 5.8 ± 0.15 | 26 | 47 |
|  |  |  | 15 | 0.25 | 16/8/0.127 | 0.85 | 6.4 ± 0.15 | 26 | 47 |
|  |  |  | 25 | 0.25 | 24/8/0.127 | 1.00 | 7.8 ± 0.20 | 26 | 47 |
|  |  |  | 37 | 0.25 | 24/8/0.127 | 1.00 | 8.9 ± 0.25 | 26 | 47 |
|  |  |  | 50 | 0.25 | 24/8/0.127 | 1.00 | 10.1 ± 0.30 | 26 | 47 |
| UL 2464 CUL AWM | 26 | 7 / 0.160 | 3 | 0.25 | 16/5/0.127 | 0.80 | 4.5 ± 0.15 | 32 | 52 |
|  |  |  | 4 | 0.25 | 16/5/0.127 | 0.80 | 4.8 ± 0.15 | 32 | 52 |
|  |  |  | 5 | 0.25 | 16/6/0.127 | 0.80 | 5.1 ± 0.15 | 32 | 52 |
|  |  |  | 6 | 0.25 | 16/6/0.127 | 0.80 | 5.4 ± 0.15 | 28 | 51 |
|  |  |  | 7 | 0.25 | 16/6/0.127 | 0.80 | 5.4 ± 0.15 | 28 | 51 |
|  |  |  | 8 | 0.25 | 16/6/0.127 | 0.80 | 5.7 ± 0.15 | 28 | 51 |
|  |  |  | 9 | 0.25 | 16/8/0.127 | 0.80 | 6.0 ± 0.20 | 28 | 51 |
|  |  |  | 10 | 0.25 | 16/8/0.127 | 0.80 | 6.3 ± 0.20 | 28 | 51 |
|  |  |  | 15 | 0.25 | 16/8/0.127 | 0.85 | 7.0 ± 0.20 | 28 | 51 |
|  |  |  | 25 | 0.25 | 24/6/0.127 | 1.00 | 8.6 ± 0.25 | 28 | 51 |
|  |  |  | 37 | 0.25 | 24/7/0.127 | 1.00 | 9.7 ± 0.25 | 28 | 51 |
|  |  |  | 50 | 0.25 | 24/8/0.127 | 1.00 | 11.0 ± 0.30 | 28 | 51 |
| UL 2464 CUL AWM | 24 | 7 / 0.203 | 3 | 0.25 | 16/5/0.127 | 0.80 | 4.7 ± 0.15 | 35 | 65 |
|  |  |  | 4 | 0.25 | 16/5/0.127 | 0.80 | 5.1 ± 0.15 | 35 | 65 |
|  |  |  | 5 | 0.25 | 16/6/0.127 | 0.80 | 5.6 ± 0.15 | 35 | 65 |
|  |  |  | 6 | 0.25 | 16/6/0.127 | 0.80 | 5.7 ± 0.15 | 30 | 65 |
|  |  |  | 7 | 0.25 | 16/8/0.127 | 0.80 | 5.7 ± 0.15 | 30 | 65 |
|  |  |  | 8 | 0.25 | 16/8/0.127 | 0.80 | 6.2 ± 0.15 | 30 | 65 |
|  |  |  | 9 | 0.25 | 16/8/0.127 | 0.85 | 6.4 ± 0.15 | 30 | 55 |
|  |  |  | 10 | 0.25 | 16/8/0.127 | 0.85 | 6.7 ± 0.20 | 30 | 55 |
|  |  |  | 15 | 0.25 | 16/8/0.127 | 0.85 | 7.4 ± 0.25 | 30 | 55 |
|  |  |  | 25 | 0.25 | 24/8/0.127 | 1.00 | 9.1 ± 0.25 | 30 | 55 |
|  |  |  | 37 | 0.25 | 24/8/0.127 | 1.00 | 10.3 ± 0.25 | 30 | 55 |
|  |  |  | 50 | 0.25 | 24/10/0.127 | 1.00 | 11.9 ± 0.30 | 30 | 55 |
| UL 2464 CUL AWM | 22 | 7 / 0.254 | 3 | 0.25 | 16/6/0.127 | 0.80 | 5.3 ± 0.15 | 37 | 67 |
|  |  |  | 4 | 0.25 | 16/6/0.127 | 0.80 | 5.6 ± 0.15 | 37 | 67 |
|  |  |  | 5 | 0.25 | 16/6/0.127 | 0.80 | 6.1 ± 0.15 | 35 | 63 |
|  |  |  | 6 | 0.25 | 16/8/0.127 | 0.80 | 6.3 ± 0.15 | 35 | 53 |
|  |  |  | 7 | 0.25 | 16/8/0.127 | 0.80 | 6.3 ± 0.15 | 35 | 63 |
|  |  |  | 8 | 0.25 | 16/8/0.127 | 0.85 | 6.9 ± 0.15 | 35 | 63 |
|  |  |  | 9 | 0.25 | 24/8/0.127 | 0.85 | 7.3 ± 0.25 | 35 | 63 |
|  |  |  | 10 | 0.25 | 24/8/0.127 | 0.85 | 7.4 ± 0.25 | 35 | 63 |
|  |  |  | 15 | 0.25 | 24/8/0.127 | 0.85 | 8.3 ± 0.25 | 35 | 63 |
|  |  |  | 25 | 0.25 | 24/8/0.127 | 1.00 | 10.6 ± 0.25 | 35 | 63 |
|  |  |  | 37 | 0.25 | 24/10/0.127 | 1.00 | 11.9 ± 0.30 | 35 | 63 |
|  |  |  | 50 | 0.25 | 24/10/0.127 | 1.00 | 13.5 ± 0.30 | 35 | 51 |
| UL 2464 CUL AWM | 18 | 34 / 0.180 | 2 | 0.40 | 16/6/0.127 | 0.80 | 6.3 ± 0.20 | 70 | 120 |
|  |  |  | 3 | 0.40 | 16/8/0.127 | 0.80 | 6.6 ± 0.20 | 70 | 120 |
|  |  |  | 4 | 0.40 | 24/8/0.127 | 0.85 | 7.3 ± 0.25 | 70 | 120 |
|  |  |  | 5 | 0.40 | 24/8/0.127 | 0.85 | 7.9 ± 0.25 | 70 | 120 |

Foil Cover Type :

| UL Style & CUL Type | Conductor |  | No. of Core | Insulation Thickness | Jacket Thickness | Overall Diameter | Nominal Capa |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | AWG | No./mm |  | mm | mm | mm | A | B |
| UL 2464 CUL AWM | 28 | 7 / 0.127 | 3 | 0.25 | 0.80 | 3.7 ± 0.15 | 28 | 51 |
|  |  |  | 4 | 0.25 | 0.80 | 3.9 ± 0.15 | 28 | 51 |
|  |  |  | 5 | 0.25 | 0.80 | 4.2 ± 0.15 | 28 | 51 |
|  |  |  | 6 | 0.25 | 0.80 | 4.5 ± 0.15 | 26 | 47 |
|  |  |  | 7 | 0.25 | 0.80 | 4.5 ± 0.15 | 26 | 47 |
|  |  |  | 8 | 0.25 | 0.80 | 4.8 ± 0.15 | 26 | 47 |
|  |  |  | 9 | 0.25 | 0.80 | 5.1 ± 0.20 | 26 | 47 |
|  |  |  | 10 | 0.25 | 0.80 | 5.2 ± 0.20 | 26 | 47 |
|  |  |  | 15 | 0.25 | 0.85 | 5.8 ± 0.20 | 26 | 47 |
|  |  |  | 25 | 0.25 | 1.00 | 7.6 ± 0.20 | 26 | 47 |
|  |  |  | 37 | 0.25 | 1.00 | 8.3 ± 0.20 | 26 | 47 |
|  |  |  | 50 | 0.25 | 1.00 | 9.5 ± 0.25 | 26 | 47 |
| UL 2464 CUL AWM | 26 | 7 / 0.160 | 3 | 0.25 | 0.80 | 3.9 ± 0.15 | 32 | 52 |
|  |  |  | 4 | 0.25 | 0.80 | 4.2 ± 0.15 | 32 | 52 |
|  |  |  | 5 | 0.25 | 0.80 | 4.5 ± 0.15 | 32 | 52 |
|  |  |  | 6 | 0.25 | 0.80 | 4.8 ± 0.15 | 28 | 51 |
|  |  |  | 7 | 0.25 | 0.80 | 4.8 ± 0.15 | 28 | 51 |
|  |  |  | 8 | 0.25 | 0.80 | 5.1 ± 0.20 | 28 | 51 |
|  |  |  | 9 | 0.25 | 0.80 | 5.4 ± 0.20 | 28 | 51 |
|  |  |  | 10 | 0.25 | 0.80 | 5.7 ± 0.20 | 28 | 51 |
|  |  |  | 15 | 0.25 | 0.85 | 6.4 ± 0.20 | 28 | 51 |
|  |  |  | 25 | 0.25 | 1.00 | 8.0 ± 0.20 | 28 | 51 |
|  |  |  | 37 | 0.25 | 1.00 | 9.2 ± 0.25 | 28 | 51 |
|  |  |  | 50 | 0.25 | 1.00 | 10.4 ± 0.25 | 28 | 51 |
| UL 2464 CUL AWM | 24 | 7 / 0.203 | 3 | 0.25 | 0.80 | 4.2 ± 0.15 | 35 | 65 |
|  |  |  | 4 | 0.25 | 0.80 | 4.5 ± 0.15 | 35 | 65 |
|  |  |  | 5 | 0.25 | 0.80 | 4.9 ± 0.15 | 35 | 65 |
|  |  |  | 6 | 0.25 | 0.80 | 5.2 ± 0.15 | 30 | 65 |
|  |  |  | 7 | 0.25 | 0.80 | 5.2 ± 0.15 | 30 | 65 |
|  |  |  | 8 | 0.25 | 0.80 | 5.5 ± 0.20 | 30 | 65 |
|  |  |  | 9 | 0.25 | 0.85 | 5.8 ± 0.20 | 30 | 55 |
|  |  |  | 10 | 0.25 | 0.85 | 6.2 ± 0.20 | 30 | 55 |
|  |  |  | 15 | 0.25 | 0.85 | 6.9 ± 0.20 | 30 | 55 |
|  |  |  | 25 | 0.25 | 1.00 | 8.6 ± 0.20 | 30 | 55 |
|  |  |  | 37 | 0.25 | 1.00 | 9.8 ± 0.25 | 30 | 55 |
|  |  |  | 50 | 0.25 | 1.00 | 11.4 ± 0.30 | 30 | 55 |
| UL 2464 CUL AWM | 22 | 7 / 0.254 | 3 | 0.25 | 0.80 | 4.7 ± 0.15 | 37 | 67 |
|  |  |  | 4 | 0.25 | 0.80 | 5.1 ± 0.15 | 37 | 67 |
|  |  |  | 5 | 0.25 | 0.80 | 5.5 ± 0.15 | 37 | 67 |
|  |  |  | 6 | 0.25 | 0.80 | 5.8 ± 0.15 | 35 | 53 |
|  |  |  | 7 | 0.25 | 0.80 | 5.8 ± 0.15 | 35 | 63 |
|  |  |  | 8 | 0.25 | 0.85 | 6.3 ± 0.15 | 35 | 63 |
|  |  |  | 9 | 0.25 | 0.85 | 6.7 ± 0.15 | 35 | 63 |
|  |  |  | 10 | 0.25 | 0.85 | 6.8 ± 0.20 | 35 | 63 |
|  |  |  | 15 | 0.25 | 0.85 | 7.8 ± 0.20 | 35 | 63 |
|  |  |  | 25 | 0.25 | 0.85 | 10.0 ± 0.20 | 35 | 63 |
|  |  |  | 37 | 0.25 | 1.00 | 11.3± 0.25 | 35 | 63 |
|  |  |  | 50 | 0.25 | 1.00 | 12.9 ± 0.30 | 35 | 63 |
| UL 2464 CUL AWM | 18 | 34 / 0.180 | 2 | 0.40 | 0.80 | 5.8 ± 0.15 | 70 | 120 |
|  |  |  | 3 | 0.40 | 0.80 | 6.0 ± 0.20 | 70 | 120 |
|  |  |  | 4 | 0.40 | 0.85 | 6.7 ± 0.20 | 70 | 120 |
|  |  |  | 5 | 0.40 | 0.85 | 7.3 ± 0.20 | 70 | 120 |

A：Capacitance between conductors

B：Capacitance between 1 conductor and other conductors connected to shield.

Note：Capacitance is measured at 1KHz