Low Voltage Computer Cable

### UL 2919

Construction :

Product Description :

Conductor

Rating temperature : UL 8O°C.

Cellular-PE Insulation

PVC Insulation

Jacket

Braid or Spiral Shield

AL-mylar foil

Drain Wire

- Rated voltage : UL 3O volts.

- Bare or tinned copper conductor.

- Two or more twisted singles or groups of twisted singles  
 twisted singles twiestd together.

- Shield and covering optional.

- PE or PVC insulation.

- PVC jacket.

Applications :

VGA MONINTOR.

| UL Style 2919 & CUL AWM | Conductor |  | No. of Core | UL Style | Insulation Thickness | No. of Core | Spiral Shield | Jacket Thickness | Overall Diameter |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | AWG | No./mm |  |  | mm | mm | No./mm | mm | mm |
| 1 + 4 | 24 | 7 / 0.203 | 4 | 1061 | 0.25 | 1.10 | AL/mylar TINNED Copper wire 95 % braid shield coverage | 1.00 | 7.8 ± 0.25 |
|  | 28 | 7 / 0.127 | 1 | 1792 | 0.50 | 2.40 |  |  |  |
| 1 + 8 | 24 | 7 / 0.203 | 8 | 1061 | 0.25 | 1.10 |  | 1.25 | 8.0 ± 0.25 |
|  | 28 | 7 / 0.127 | 1 | 1792 | 0.50 | 2.40 |  |  |  |
| 3 + 4 | 24 | 7 / 0.203 | 4 | 1061 | 0.25 | 1.10 |  | 1.50 | 8.0 ± 0.25 |
|  | 28 | 7 / 0.127 | 3 | 1792 | 0.50 | 2.40 |  |  |  |
| 3 + 5 | 24 | 7 / 0.203 | 5 | 1061 | 0.25 | 1.10 |  | 1.05 | 8.15 ± 0.25 |
|  | 28 | 7 / 0.127 | 3 | 1792 | 0.50 | 2.40 |  |  |  |